# Developer’s Guide to the Hampod 2.0

This guide provides detailed instructions for setting up your NanoPi Neo from scratch and details for developers on expanding the current design. The methodology in this guide is recommended if you want to have the most up to date software version for your Hampod. For most users, we recommend following the “Getting Started” section of the user manual.

This document will cover the in-depth setup of the Hampod from the preparation of a fresh Debian OS image on your SD card to booting up the board for the first time and deploying your code followed by detailed descriptions of the current code base, operations, and specific areas for improvement in the software at time of writing. It will also provide descriptions of how to add additional functionality in the software for more Hamlib functions, as well as how to tie them to the additional unused keys on the keypad.

## 1. Required Equipment

Before you begin, ensure you have the following items:

- NanoPi Neo Board: The single-board computer you wish to set up.

- microSD Card: A Class 10 card with a minimum of 8GB capacity (16GB or larger is recommended for extra space in order to store all necessary audio files).

- microSD Card Reader: To connect the microSD card to your Windows PC.

- Power Supply: A stable 5V power adapter with sufficient amperage (2A but you might be able to get away with less).

- Ethernet Cable: For network connectivity; the NanoPi Neo is typically headless, so a wired connection is required for uploading new code or running nonstandard software.

- MicroUSB: To send power from your PC to the NanoPi Neo

- Optional – USB-to-Serial Adapter: To connect to the UART pins located on the compute board if network access isn’t available.

- Optional – USB Keyboard and Monitor: If you plan to use a local console (though NanoPi Neo is designed primarily for headless operation and does not have a built-in display output).

## 2. Software Needed

On a separate computer, install the following software:

- Imaging Tool:

  - Balena Etcher or Win32DiskImager to flash the OS image onto the microSD card. This will allow you to either start over if things go wrong or upload new images you create.

- Terminal/SSH Client:

  - PuTTY (or any SSH client) to connect to the NanoPi Neo once it’s booted.

    - Putty will work if using a USB-to-serial adapter for direct console access

- File Transfer Utility:

  - WinSCP for transferring code files between your PC and the NanoPi Neo (alternatively, you can use Git if the board has internet access). A regular scp command should also work.

## 3. Downloading the Debian OS Image

- Locate the Image:

Follow the link provided to obtain the Debian OS image and download ‘h3_sd_debian-jessie_4.14_armhf_20191219.img.gz’:

https://dl.friendlyelec.com/nanopineo

https://drive.google.com/drive/folders/1rcF2-Qrw9o_OACNV6cAj_Vzv-2BYDGQz?usp=drive_link

## 4. Flashing the OS Image onto the microSD Card

- Insert the microSD Card:
Place your microSD card into the card reader and connect it to your computer.

- Launch Your Imaging Tool:
Open Balena Etcher (or Win32DiskImager).

  - Select Image: Browse and select the downloaded Debian OS image.

  - Select Target: Choose the microSD card as the destination.

  - Flash: Start the flashing process. This process may take several minutes.

- Completion Check:
Once the flashing is complete be sure to safely eject the microSD card from your computer.

## 5. First Boot and Initial Configuration

- Insert the microSD Card:
Place the freshly imaged microSD card into the NanoPi Neo’s SD card slot.

- Connect Peripherals:

  - Plug in the Ethernet cable to enable network connectivity.

  - Optionally connect your USB-to-serial adapter if you plan to debug via serial console.

  - Plug in the microUSB into your PC and connect it to the NanoPi Neo

- Power On:
The NanoPi Neo should power on automatically after being connected with the microUSB. You should observe LED indicators on the board that signify boot activity. The board will automatically boot using the image on the microSD card.

- Determine IP Address:

  - If connected to a network with DHCP, check your router’s admin panel or use an IP scanner to find the NanoPi Neo’s assigned IP address.

  - Alternatively, if using a serial console, messages displayed during boot may include network details.

- SSH into the NanoPi Neo:

  - Open PuTTY on your Windows PC.

  - Enter the IP address of the NanoPi Neo and connect via SSH.

  - Alternatively, if using a serial console, select the appropriate COM port and set the baud rate to 115200

  - Use the default login credentials provided in the board’s documentation (for the specified image file, the username is root, and the password is fa).

## 6. Deploying Your Code

Once you’re logged in:

- Method 1: Using Git (Recommended)

  - Install Git:
If Git isn’t pre-installed, update your package list and install it:

bash

Copy

apt-get update
apt-get install git

  - Clone Your Repository:
Clone your code repository:

bash

Copy

git clone git@github.com:rhit-trinhmv/Team-1-HAMPOD2.git

  - Important note when cloning the repository this way:

    - This copies all the source files and all the hardware files to your nanopi, if you have a smaller SD card, this may cause issues.

- Method 2: Using SCP/WinSCP

  - Setup File Transfer:
Use WinSCP on your Windows PC to connect via SFTP or SCP to your NanoPi Neo using the IP address and login credentials.

  - Download project files:

Go to https://github.com/rhit-trinhmv/Team-1-HAMPOD2/tree/main# and download the source code.

  - Transfer Files:
Drag and drop the Firmware and Software folders from the Hampod2 github to the desired directory on the NanoPi Neo (default is root.)

- Method 3: USB Drive (Alternative)

  - Download project files:

Go to https://github.com/rhit-trinhmv/Team-1-HAMPOD2/tree/main# and download the source code.

  - Copy Code to USB:
Copy your code files to a USB flash drive.

  - Mount USB Drive:
Insert the USB drive into the NanoPi Neo (if supported) and mount it manually:

bash

Copy

mkdir /mnt/usb
mount /dev/sda1 /mnt/usb

  - Copy Files:
Transfer the Firmware and Software folders from the Hampod2 github to the desired directory on the NanoPi Neo (default is root.)

## 7. Running Your Code

Once you’ve copied the files from the repository:

- “cd” into the folder named “Team-1-Hampod2"

- Give yourself proper permissions to run the startup file (may not be necessary)

bash

Copy

chmod +x startup.sh

- Run the startup file

bash

Copy

./startup.sh

## 8. Common Issues and Troubleshooting

### Issue 1: microSD Card Not Booting

- Potential Causes:

  - Incorrect or corrupted OS image.

  - Faulty microSD card.

- Solution:

  - Verify the image checksum.

  - Re-flash the image with a reliable tool.

  - Try using a different microSD card.

### Issue 2: Network Connectivity Problems

- Potential Causes:

  - Ethernet cable not connected securely.

  - DHCP not assigning an IP address.

- Solution:

  - Ensure the Ethernet cable is firmly connected.

  - Check your router’s DHCP settings.

  - If necessary, configure a static IP via the serial console.

### Issue 3: SSH Connection Refused/Timeout

- Potential Causes:

  - The board may not have booted completely.

  - Incorrect IP address.

- Solution:

  - Wait a few minutes after booting and check the board’s LEDs or serial console for boot completion.

  - Recheck your network’s DHCP client list to confirm the correct IP.

### Issue 4: USB-to-Serial Adapter Drivers Not Installed

- Potential Causes:

  - Windows may lack the appropriate drivers.

- Solution:

  - Download and install the latest drivers for your specific USB-to-serial adapter from the manufacturer’s website.

### Issue 5: Code Transfer Fails

- Potential Causes:

  - Firewall or network issues blocking SCP/SSH.

  - Not enough space on the microSD card

- Solution:

  - Ensure your Windows firewall or antivirus software is not interfering with your SSH/SCP client.

  - Test the connection using a different method (e.g., USB drive) to isolate the issue.