Hampod Accessible User Guide

This guide is designed to help you operate the Hampod device using simple, step-by-step instructions. It explains how to change between modes, what each mode does, and how to use Normal Mode to access a variety of radio control functions using specific keypad button presses. Follow the spoken instructions and use the keypad as directed to complete each action.

1. POWERING ON THE DEVICE

Step-by-Step Instructions

- Connect the Device:

  - Need: 5V 2A barrel jack connector

  - Action: Using the 5V AC To DC Power Supply, plug the power supply into an outlet, and connect it with Hampod

- Establish a Serial Connection:

  - Action: Using the USB to Micro USB Cable, plug Micro USB end into the Micro USB port on the Hampod, and plug the USB end into a laptop or desktop

  - Action: On your laptop or desktop, open your preferred terminal application.

  - Action: Connect to the device serially using its MAC address.

  - Audio Feedback: The device will confirm the connection with a spoken message, “Hampod is now connected.”

- Power On the Device:

  - Action: Once connected properly the device will be automatically powered on.

  - Audio Feedback: You will hear an audio statement letting you know the Hampod is now running.

- Wait for the Startup Prompt:

  - Listen: The device will announce whether you want to load a saved configuration or set up a new rig.

  - You Will Hear: “Press 0 for saved configuration or 1 for new setup.”

Overview of Modes and How to Change Between Them

Your Hampod device operates in several distinct modes. Each mode provides different functions:

- Normal Mode (Mode 1):
This is the default mode. In Normal Mode, you can access many radio functions such as retrieving settings, getting or setting parameters (like noise reduction, compression, tone control, VFO selection, etc.), and viewing the current configuration.

- Frequency Mode (Mode 2):
Allows you to change the operating frequency by entering numeric values.

- DTMF Mode (Mode 3):
Used for sending tone commands to an external radio via the keypad.

- Configuration Mode (Mode 4):
Enables you to view and change the Hampod’s settings and save your rig configuration.

- Monitoring Mode (Mode 5):
Provides continuous real-time status updates (such as current frequency and signal strength).

Changing Modes

- Enter Mode Select:

  - Action: Press and hold the [B] button for one full second.

  - Audio Feedback: The device will announce, “Mode Select activated.”

- Select a Mode:

  - Action: While still in Mode Select, press the following number keys for the corresponding mode:

    - Press 1 for Config Mode.

    - Press 2 for Normal Mode.

    - Press 3 for DTMF Mode.

    - Press 4 for Monitoring Mode.

    - Press 5 for Frequency Mode.

  - Audio Feedback: The device will announce your selection (for example, “Normal Mode active”).

2. OPERATING IN NORMAL MODE

Normal Mode is the default mode for radio control. In this mode, you can use specific keypad commands to access various functions. The functions are activated using a combination of shift commands and direct key presses. For each function, you will receive spoken instructions that guide you through the process.

How Normal Mode Works

- Shift Commands:
Use the [A] key to cycle through available functions. Each press of [A] advances the shift value (S0, S1, S2, etc.).

- Direct Key Actions:
After setting the desired shift value, press a specific number key to either retrieve or set a parameter.

- Hold Actions:
If a function requires setting a value, you may be instructed to press and hold a number key. When you release, the device will register your input.

- Reset:
Once a function is executed, the shift value resets to 0. You will need to press [A] again to re-select a function if necessary.

Step-by-Step Instructions for Normal Mode Functions

A. Retrieving a Parameter (Example: Noise Reduction)

- Activate Normal Mode:

  - Action: Ensure that the device is in Normal Mode.

  - Audio Feedback: You should hear, “Normal Mode active. Ready for command.”

- Select the Function:

  - Action: Press the [A] key once to enter shift level S0.

  - Audio Feedback: The device will state, “Shift level zero. Available functions: Noise Reduction, VFO selection, etc.”

- Retrieve Noise Reduction:

  - Action: Press the number key corresponding to noise reduction (for example, if the audio instruction says “Press 0 for Noise Reduction,” press 0).

  - Audio Feedback: The device announces, “Noise Reduction is set to [value].”

B. Setting a Parameter (Example: Setting Tone Control)

- Activate Normal Mode:

  - Action: Confirm that you are in Normal Mode.

  - Audio Feedback: “Normal Mode active. Ready for command.”

- Select the Function:

  - Action: Press the [A] key twice to move to shift level S2.

  - Audio Feedback: The device will say, “Shift level two. Tone Control function selected.”

- Enter the New Value for Tone Control:

  - Action: Press and hold the number key that corresponds to the desired tone control adjustment (for example, if instructed, press and hold 0).

  - Audio Feedback: You will hear, “Entering Tone Control adjustment mode. Release when value is reached.”

  - Action: Release the key when you have reached your desired setting.

  - Audio Feedback: The device announces, “Tone Control set to [value].”

C. Retrieving Other Parameters (Example: VFO Selection)

- Activate Normal Mode:

  - Action: Confirm that you are in Normal Mode.

  - Audio Feedback: “Normal Mode active. Ready for command.”

- Select the Function:

  - Action: Press the [A] key once or as instructed until you reach the function for VFO selection.

  - Audio Feedback: The device will announce, “VFO selection function active.”

- Retrieve Current VFO:

  - Action: Press the designated number key for getting the current VFO (for example, press 1 if that is instructed).

  - Audio Feedback: The device announces, “Current VFO is [A/B/C].”

D. Accessing Other Functions

For each additional function available in Normal Mode (such as getting compression settings, checking RIT status, etc.), follow these general steps:

- Set the Appropriate Shift Level:

  - Action: Press the [A] key the required number of times until the device announces the desired function (e.g., “Compression function selected”).

- Press the Designated Number Key:

  - Action: Press the specific number key as instructed (e.g., press 1 for retrieving compression).

- Listen for Confirmation:

  - Audio Feedback: The device will read out the current value or confirm the setting.

Note: The exact key mappings (such as which number corresponds to which function) will be provided in the audio prompts when you are in Normal Mode. Follow the spoken instructions carefully.

3. FREQUENCY MODE – CHANGING THE FREQUENCY

Frequency Mode lets you change the operating frequency using only the keypad.

How to Change the Frequency Step by Step

- Switch to Frequency Mode:

  - Action: Press and hold [B] for one second, then press 5.

  - Audio Feedback: “Frequency Mode active.”

- To Enter the Frequency:

Below is an example of entering 43.575 MHz

  - Step 1: Press the first digit (e.g., press 4).

  - Step 2: Press the next digit (e.g., press 3).

  - Step 3: Press the [*] key to add a decimal point.

  - Step 4: Press the next digit (e.g., press 5).

  - Step 5: Press the following digit (e.g., press 7).

  - Step 6: Press the final digit (e.g., press 5).

  - Step 7: Press [#] to confirm

  - Audio Feedback: “Frequency updated to 43.575 MHz” (or your entered value).

4. DTMF MODE – SENDING TONE COMMANDS

DTMF Mode is used to send tone commands.

Sending a DTMF Command

- Switch to DTMF Mode:

  - Action: Press and hold [B] for one second, then press 3.

  - Audio Feedback: “DTMF Mode active.”

- Enter the Command:

  - Action: Use the numeric keypad to enter the specific DTMF sequence as instructed by the audio prompts.

  - Audio Feedback: Listen for a tone and the confirmation message, “DTMF command sent.”

5. CONFIGURATION MODE – CHANGING SETTINGS AND SAVING CONFIGURATIONS

Configuration Mode lets you modify settings and save your rig configuration.

Changing a Setting

- Enter Configuration Mode:

  - Action: From Normal Mode, press and hold [B] for one second, then press 4.

  - Audio Feedback: “Configuration Mode active.”

- Navigate Settings:

  - Action:

    - Press [4] to go to the previous setting.

    - Press [6] to go to the next setting.

    - Press [2] to increase the current setting’s value.

    - Press [8] to decrease the current setting’s value.

    - Press [9] to confirm setting changes.

  - Audio Feedback: The device will read out the setting name and its current value as you navigate.

Saving Your Rig Configuration

- While in Configuration Mode:

  - Action: Press [5] to save the current configuration.

  - Audio Feedback: “Configuration saved.”

6. MONITORING MODE – RECEIVING STATUS UPDATES

Monitoring Mode provides continuous, real-time status updates.

Activating Monitoring Mode

- Switch to Monitoring Mode:

  - Action: Press and hold [B] for one second, then press 4.

  - Audio Feedback: “Monitoring Mode active.”

- Listen for Updates:

  - Action: Remain in this mode.

  - Audio Feedback: The device will automatically announce current parameters such as frequency and signal strength at regular intervals.

7. TROUBLESHOOTING AND HELP

- No Audio Feedback:

  - Check: Ensure that the volume is turned up.

  - Action: Verify that your headphones or speakers are properly connected.

- Unclear Instructions:

  - Action: If you miss a spoken prompt, press and hold the [B] button to enter Mode Select again, then select your desired mode.

- General Issues:

  - Reference: Consult the printed or digital manual for additional troubleshooting tips.

FINAL REMINDERS

- Listen Carefully:
Always pay attention to the spoken feedback after each action.

- Follow Every Step:
Do not skip any instructions. Each step is designed to ensure safe and effective operation.

- Return to Normal Mode if Needed:
If you become confused, press and hold [B] for one second, then press 2 to return to Normal Mode.

- Take Your Time:
Work through each step at a comfortable pace. The device will repeat important instructions if necessary.

This guide is intended to make operating your Hampod device as simple as possible. Follow the detailed instructions, listen for the spoken feedback, and use the keypad commands exactly as described.

Happy operating!