# Raspberry Pi Development Setup Guide

This guide provides detailed, step-by-step instructions for setting up a Raspberry Pi (RPi) from scratch to run the HAMPOD project.

Note: This guide focuses on installing the code and dependencies directly on the Raspberry Pi. For a standard development workflow, you will likely also want to clone the repository on your Development Machine (PC/Mac) to edit files locally and then sync them to the Pi for testing (e.g., using the remote_install.sh script mentioned in the Tips section). Steps for setting up your local Development Machine environment are not covered in detail here.

## 📋 Prerequisites

Before you begin, ensure you have the following:

- Raspberry Pi: Recommended RPi 5 or RPi 4 (4GB+ RAM preferred, though 2GB works).

- microSD Card: 16GB or larger (Class 10 / UHS-1 recommended).

- Power Supply: Official USB-C power supply for your specific RPi model.

- Computer: A Windows, Mac, or Linux computer to flash the SD card.

- Internet Connection: Wi-Fi credentials or an Ethernet cable.

## 🚀 Step 1: Install Debian Trixie

The project is currently being developed on Debian Trixie. Please install this version using the Raspberry Pi Imager.

- Download & Install Raspberry Pi Imager:

  - Go to raspberrypi.com/software and download the imager for your computer.

- Configure the Image:

  - Open Raspberry Pi Imager.

  - Choose Device: Select your RPi model (e.g., Raspberry Pi 5).

  - Choose OS:

    - Click “Choose OS”.

    - Select General Purpose OS -> Debian -> Debian Trixie (or the specific Trixie image if listed differently).

  - Choose Storage: Select your microSD card.

- Advanced Options (Crucial Step):

  - Click “Next”. When asked “Would you like to apply OS customisation settings?”, verify EDIT SETTINGS.

  - General Tab:

    - Set hostname: HAMPOD (or whatever you prefer, this guide assumes HAMPOD.local).

    - Set username and password:

      - Username: hampoduser

      - Password: Choose a secure password.

    - Configure Wireless LAN: Enter your Wi-Fi SSID and password (if not using Ethernet).

  - Services Tab:

    - Enable SSH: Check “Use password authentication”.

  - Click SAVE and then YES to apply.

- Write and Verify:

  - Click YES to begin writing. This will erase the SD card.

  - Once finished, insert the SD card into your Raspberry Pi and power it on.

## 🔌 Step 2: Connect via SSH

Give your Pi a minute or two to boot up and connect to the network.

- Open a terminal (Command Prompt or PowerShell on Windows, Terminal on Mac/Linux).

- (DEV:) Connect using the hostname and username you set: bash     ssh hampoduser@HAMPOD.local (If prompted about authenticity, type yes)

- Enter your password when prompted.

Troubleshooting: If HAMPOD.local doesn’t work, check your router’s device list to find the IP address of the Pi and use ssh hampoduser@192.168.x.x instead.

### Optional: Set up an SSH Key

To avoid typing your password every time, you can generate an SSH key on your computer and copy it to the Pi.

- (DEV:) Generate a key (if you don’t have one): On Windows (PowerShell) or Mac/Linux (Terminal): bash     ssh-keygen -t ed25519 (Press Enter to accept defaults. If using Windows Command Prompt, ensure OpenSSH is installed/enabled)

- (DEV:) Copy the key to the Pi: On Windows (PowerShell): powershell     type $env:USERPROFILE\.ssh\id_ed25519.pub | ssh hampoduser@HAMPOD.local "mkdir -p ~/.ssh && cat >> ~/.ssh/authorized_keys" On Mac/Linux: bash     ssh-copy-id hampoduser@HAMPOD.local

- Verify: Log out and log back in. You should not be asked for a password.

## 🛠️ Step 3: Install System Dependencies

Once logged in, update the system and install the required build tools and libraries.

- (RPI:) Update Package Lists: bash     sudo apt update && sudo apt upgrade -y

- (RPI:) Install Development Tools & Audio Libraries: HAMPOD requires git, make, gcc, ALSA (audio), and Hamlib (rig control). bash     sudo apt install -y git make gcc libasound2-dev libhamlib-dev

## 📂 Step 4: Clone the HAMPOD Repository

Now, download the project code to your home directory.

- (RPI:) Clone the Repository: > Note: The HAMPOD2026 repository is public, so you do not need a GitHub account simply to clone and run it. You only need a GitHub account if you plan to contribute changes back to the project. If you plan to contribute and don’t have an account, you can create one for free at github.com/join. bash     cd ~     git clone https://github.com/waynepadgett/HAMPOD2026.git

- (RPI:) Enter the Directory: bash     cd HAMPOD2026

## 🗣️ Step 5: Install Piper TTS

HAMPOD uses Piper for high-quality, low-latency text-to-speech. We have a script to automate this installation.

- (RPI:) Make Scripts Executable: bash     chmod +x Documentation/scripts/*.sh

- (RPI:) Run the Installer: bash     ./Documentation/scripts/install_piper.sh This script downloads the Piper binary, installs it to /usr/local/bin/piper, and downloads the required voice model (en_US-lessac-low.onnx) to Firmware/models/.

## 🏗️ Step 6: Build the Project

Currently, the primary component to build is the Firmware (which handles hardware, audio, and keypad input).

Note on Software Layer: The high-level Software layer is currently under active development (“Fresh Start” refactoring in Software2/). The legacy Software/ directory may not build cleanly on a fresh system. For now, focus on building the Firmware and running the HAL Integration Test (see Step 7) to verify your setup.

### Build Firmware (RPI:)

cd ~/HAMPOD2026/Firmware
make

If successful, you will see a firmware.elf file created.

## ▶️ Step 7: Verify Your Setup (HAL Integration Test)

The recommended way to verify your Raspberry Pi setup is to run the HAL Integration Test. This standalone test verifies that the keypad and audio/TTS subsystems are working correctly.

- (RPI:) Build the Test: bash     cd ~/HAMPOD2026/Firmware/hal/tests     make

- (RPI:) Run the Test (from the Firmware directory, required for model paths): bash     cd ~/HAMPOD2026/Firmware     sudo hal/tests/test_hal_integration

- What to Expect:

  - The system should announce “System Ready” (or similar) via the speaker.

  - Pressing keys on the keypad should announce the key name (e.g., “One”, “Enter”).

  - Use Ctrl+C to exit.

  - This confirms that your hardware, audio drivers, and Piper TTS are all correctly configured.

If you only hear a click sound (no speech), see the Troubleshooting Audio/TTS section below.

### Running the Full System (Advanced)

Once the Software layer development is complete, you will run Firmware and Software in separate terminals:

- (RPI:) Start Firmware: bash     cd ~/HAMPOD2026/Firmware     sudo ./firmware.elf

- (RPI:) Start Software (in a new SSH session): bash     cd ~/HAMPOD2026/Software2     ./bin/hampod Note: The Software layer (Software2/) is under active development. Check its README for current build instructions.

## 🔧 Troubleshooting Audio/TTS

If the integration test runs but you only hear a click sound (no spoken words), the issue is likely with Piper TTS or the audio configuration.

### 1. Verify Piper is Installed (RPI:)

which piper
piper --version

If piper is not found, re-run the installer:

./Documentation/scripts/install_piper.sh

### 2. Verify Voice Model Exists (RPI:)

The voice model files must be in ~/HAMPOD2026/Firmware/models/:

ls -la ~/HAMPOD2026/Firmware/models/

You should see en_US-lessac-low.onnx and en_US-lessac-low.onnx.json. If missing, re-run the installer script.

### 3. Test Piper Manually (RPI:)

Test that Piper can generate and play speech:

echo "Hello World" | piper --model ~/HAMPOD2026/Firmware/models/en_US-lessac-low.onnx --output_raw | aplay -r 16000 -c 1 -f S16_LE

- If you hear “Hello World”: Piper is working. The issue may be in how the integration test invokes it.

- If you hear nothing: Continue to step 4.

### 4. Test Basic Audio Output (RPI:)

Verify that the audio hardware itself is working:

# List available audio devices
aplay -l

# Play a test tone (you should hear a voice saying "Front Left", "Front Right")
speaker-test -c 2 -t wav

If no audio is heard, check: - Is the speaker/headphones connected and powered on? - Is the correct audio output selected? (HDMI vs. headphone jack vs. USB audio) - Try forcing audio output: sudo raspi-config → System Options → Audio.

### 5. Check User Permissions (RPI:)

Ensure your user is in the audio group:

groups

If audio is not listed, add yourself:

sudo usermod -aG audio $USER

Then log out and log back in for the change to take effect.

## 📝 Tips for Developers

- Remote Install Script: (DEV:) For rapid development, use the remote_install.sh script from your local machine to push code and rebuild on the Pi automatically.

- # From your local machine
./Documentation/scripts/remote_install.sh "Update description"

- Cleaning Builds: (RPI:) If you run into weird issues, try cleaning:

- make clean
make

- Process Cleanup (After a crash): (RPI:) If the firmware or software crashes, it might leave “ghost” processes or named pipes that prevent restarting. Run this to clean up:

- # Kill lingering processes
sudo pkill -9 firmware
sudo pkill -9 imitation_software
pkill -9 Software.elf

# Remove stale pipes
rm -f ~/HAMPOD2026/Firmware/Firmware_i
rm -f ~/HAMPOD2026/Firmware/Firmware_o

- Remote Reboot: (RPI:) If the system becomes unresponsive or audio drivers lock up:

- sudo reboot